Experiment Number: 06

Name of the Experiment:

Verification of Thevenin’s Theorem and Maximum Power Transfer Theorem.

Objective:

To Verify the Thevenin’s theorem.

Theory:

Thevenin’s theorem states that a linear two-terminal circuit can be replaced by an equivalent circuit consisting of a voltage source VTh in series with a resistor RTh,

where VTh is the open-circuit voltage at the terminals and

RTh is the input or equivalent resistance at the terminals when the independent sources are turned off.

Circuit Diagram

Fig.6.1: Measurement of Vth or Voc .

Fig. 6.3.: Measurement of IL ( IL = VTh / ( RTh + RL) ).

Fig. 6.2.: Measurement of RTh.

# Required Equipment:

- Multimeter

- Resistors

- Bread Board

- Connecting Wires

- Power Supply

# Procedure:

- Connect the circuit diagram as shown in fig.6.1

- Measure current in RL.

- Connect the circuit as shown in fig. 6.2.

- Measure open circuit voltage Voc by open circuiting terminals i.e, Vth

- Draw the Thevenin’s equivalent circuit as shown in fig6.3

- Measure current flows through RL

# Precautions

- Follow the manufacturer’s instructions and lab manual for using the equipment. Directions in the user manual provide recommended operating procedures and safety precautions.

- Never hurry. Haste causes many accidents.

- Connect the power source last. Disconnect the power source first.

- Use safety goggles if necessary.

- Notify lab assistant immediately if any injuries or hazard is happened

- Before connecting the power, check your wiring carefully for agreement with the wiring diagram for an accidental short-circuit and for loose connections.

# Data Table:

| Parameters | Theoretical Values | Practical Values |
| --- | --- | --- |
| Voc |  |  |
| Rth |  |  |
| IL |  |  |

Data analysis:

- Analyze the obtained data and plot the figures if needed.

- Compare the simulation results with your experimental data /wave shapes and comment on the difference (If any).

# Results and Discussion:

- Interpret the data /findings and determine the extent to which the experiment was successful in complying with the goal that was initially set.

- Detect the variation between theory and experimental result if any and propose the ways

/procedures to overcome the variations.

- State the particular steps adopted to overcome the deviations between theory and experiment.

# Conclusion:

Summarize the experiment and discuss whether the objectives were fulfilled or not with in a short paragraph.

# Reference

- Robert L. Boylestad , “Introductory circuit analysis’’, prentice Hall , 12th Edition , Newyork, 2010, ISBN 9780137146666.

- Charles Alexander and Matthew Sadiku , “Fundamentals of Electric Circuits’’ 6th Edition ISBN 9780078028229 .

# Pre-Lab Viva Questions:

  - What is load resistance?

  - How will you calculate Thevenin’s resistance?

  - How will you calculate Thevenin’s voltage?

  - How will you calculate load current IL?

# Post Lab Viva Questions:

- Write the applications of Thevenin’s theorem.

- Write the limitations of Thevenin’s theorem.

- Verification of Maximum Power Transfer Theorem.

Objective: To design the load resistor which absorbs maximum power from source.

Theory:

The maximum power transfer theorem states that maximum power is delivered from a source to an load resistance when the load resistance is equal to source resistance. (RL = Rs is the condition required for maximum power transfer).

# Circuit Diagram:

Fig. 6.2: Maximum Power Transfer Circuit

# Required Equipment:

- Multimeter

- Resistors

- Bread Board

- Connecting Wires

- Power Supply

# Procedure:

- Connect the circuit as shown in fig.6.2

- Vary the load resistance in steps and note down voltage across the load and current flowing through the circuit.

- Calculate power delivered to the load by using formula P=V*I.

- Draw the graph between resistance and power (resistance on X- axis and power on Y-axis).

- Verify the maximum power is delivered to the load when RL = Rs for DC.

# Precautions

- Follow the manufacturer’s instructions and lab manual for using the equipment. Directions in the user manual provide recommended operating procedures and safety precautions.

- Never hurry. Haste causes many accidents.

- Connect the power source last. Disconnect the power source first.

- Use safety goggles if necessary.

- Notify lab assistant immediately if any injuries or hazard is happened

- Before connecting the power, check your wiring carefully for agreement with the wiring diagram for an accidental short-circuit and for loose connections.

# Data Table:

| S. No | RL | V | I | P=VI |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |

Graph:

# Data analysis:

- Analyze the obtained data and plot the figures if needed.

- Compare the simulation results with your experimental data /wave shapes and comment on the difference (If any).

# Results and Discussion:

- Interpret the data /findings and determine the extent to which the experiment was successful in complying with the goal that was initially set.

- Detect the variation between theory and experimental result if any and propose the ways

/procedures to overcome the variations.

- State the particular steps adopted to overcome the deviations between theory and experiment.

# Conclusion:

Summarize the experiment and discuss whether the objective were fulfilled or not within a short paragraph.

# Reference

- Robert L. Boylestad , “Introductory circuit analysis’’, prentice Hall , 12th Edition , Newyork, 2010, ISBN 9780137146666.

- Charles Alexander and Matthew Sadiku , “Fundamentals of Electric Circuits’’ 6th Edition ISBN 9780078028229 .

# Pre-Lab Viva Questions:

- State maximum power transfer theorem.

- Is it possible to apply maximum power transfer theorem to ac as well as dc circuit?

- How to find power using maximum power transfer theorem?

# Post Lab Viva Questions:

- What are conditions for maximum power transfer theorem?

- Is it possible to apply maximum power transfer theorem to nonlinear circuit?